SIMATS ENGINEERING

Department of Computer Science & Engineering ASSESSMENT TOOL 2-COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | V.Rajitha | Reg.No.: | 192311036 |
| Department: |  | Date: |  |

ANNEXURE A

COMPARATIVE ANALYSIS

1.Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

2.Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

3.Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

4.Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

5.Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

Code of Conduct:

I V.Rajitha(192311036) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: V.Rajitha

MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

1.Comparison of L1, L2, and L3 Cache

Cache memory is a small, high-speed memory located close to the CPU. It stores frequently used data and instructions to reduce the time required to access main memory.

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Size | Smallest (typically KBs) | Medium (hundreds of KBs–few MBs) | Largest (several MBs–tens of MBs) |
| Latency | Lowest (~1–4 cycles) | Low (~5–15 cycles) | Higher (~20–50+ cycles) |
| Bandwidth | Highest | High | Lower than L1/L2 |
| Throughput | Highest | High | Moderate |
| Location | Inside each CPU core | Usually per core | Usually shared among cores |
| Access speed | Fastest | Fast | Slowest among caches |
| Purpose | Immediate/frequent data | Recently used data | Shared data and larger working sets |

Impact on Processor Performance:

- L1 Cache

  - Provides the fastest access to frequently used instructions and data.

  - Its low latency allows the CPU to continue execution with minimal waiting.

  - A high L1 hit rate significantly improves instruction throughput.

- L2 Cache

  - Acts as a backup when data is not found in L1.

  - It is larger but slower than L1.

  - It reduces the number of expensive accesses to RAM.

- L3 Cache

  - Usually shared by multiple CPU cores.

  - Its larger capacity helps store data that cannot fit into L1 or L2.

  - It reduces main-memory traffic, especially in multicore workloads, but has higher latency.

## L1, L2, and L3 Cache Comparison:

| Feature | L1 | L2 | L3 |
| --- | --- | --- | --- |
| Size | 32–64 KB | 256 KB–2 MB | 4–64+ MB |
| Latency | ~1–4 cycles | ~10–20 cycles | ~30–70 cycles |
| Bandwidth | Highest | Medium | Lowest (per-core) |
| Location | Per-core | Per-core (usually) | Shared across cores |
| Role | Feeds CPU directly | Buffers L1 misses | Last stop before RAM |

Performance impact: Latency increases and bandwidth decreases from L1 → L3 → RAM. Higher hit rates at L1/L2 boost speed; misses that escalate to L3/RAM cause pipeline stalls. Shared L3 can face contention in multi-core workloads.

L1, L2, L3 Cache Comparison*

Feature ¦ L1 Cache ¦ L2 Cache ¦ L3 Cache

Location ¦ Inside each core, closest to ALU ¦ Inside each core or shared per core, after L1 ¦ Shared among all cores, outside cores

Size ¦ Smallest: 32KB - 64KB per core (Split as L1i, L1d) ¦ Medium: 256KB - 1MB per core ¦ Largest: 8MB - 32MB+ shared

Latency ¦ Lowest: ~1-4 CPU cycles, ~1ns ¦ Medium: ~10-15 cycles, ~3-5ns ¦ Highest: ~30-60 cycles, ~10-20ns

Bandwidth / Throughput ¦ Highest: Can provide 2-3 accesses per cycle, highest throughput ¦ Moderate: Lower than L1 but higher than L3 ¦ Lowest: Shared bus, contention from all cores

Hit Rate ¦ ~80-90% but smallest ¦ ~90-95% ¦ ~95%+ if present

2. Impact on Processor Performance:

1.  Latency Impact:  CPU runs at GHz speed, RAM takes ∼200-300 cycles. L1 provides data almost instantly. If L1 misses, processor stalls and checks L2, then L3. Lower L1 latency directly determines IPC - Instructions Per Cycle. High L1 miss rate causes pipeline stalls.

2. Bandwidth & Throughput Impact: * L1 has highest bandwidth to feed superscalar execution units that need multiple operands per cycle. L2 acts as buffer so L1 miss does not go to slow L3/Main memory. L3 throughput limits multi-core performance, if all cores request L3 simultaneously, bandwidth becomes bottleneck.Bottom of Form

3.Overall Hierarchy Effect:

It works on principle of locality. L1 captures temporal and spatial locality for immediate execution. L2 captures working set of current thread. L3 captures shared data and prevents costly DRAM access.

# 2. SRAM vs DRAM :

| # Feature | # SRAM | # DRAM |
| --- | --- | --- |
| # Full Form | # Static Random Access Memory | # Dynamic Random Access Memory |
| # Storage Cell | # Flip-flop (typically 6 transistors) | # Capacitor + transistor |
| # Latency | # Very low | # Higher |
| # Bandwidth | # High | # High, but generally lower effective latency |
| # Throughput | # Very high for low-latency CPU access | # High for bulk memory transfers |
| # Refresh | # Not required | # Required periodically |
| # Power | # Higher per bit | # Lower per bit |
| # Cost | # Expensive | # Cheaper |
| # Capacity | # Smaller | # Much larger |
| # Typical Use | # CPU L1, L2, L3 cache | # Main memory (RAM) |

# 1. SRAM in Cache Design

# SRAM provides very low latency, allowing the CPU to access frequently used data quickly.

# It has high bandwidth and can support rapid repeated accesses.

# Since SRAM is expensive and occupies more chip area, cache sizes are relatively small.

# Therefore, SRAM is ideal for L1, L2, and L3 caches.

# 2. DRAM in Main Memory

# DRAM has higher latency because its cells must be accessed and periodically refreshed.

# It offers high memory capacity at a much lower cost per bit.

# Modern DRAM systems use techniques such as multiple banks, burst transfers, and wider memory channels to achieve high bandwidth and throughput.

# Therefore, DRAM is mainly used as system/main memory.

SRAM vs DRAM

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Structure | 6 transistors/bit, no capacitor | 1 transistor + 1 capacitor/bit |
| Used in | Cache (L1, L2, L3) | Main memory (RAM) |
| Latency | Very low (~1–10 ns) | Higher (~50–100 ns) |
| Bandwidth | High (wide, on-chip buses) | Lower (narrower, off-chip bus) |
| Throughput | High — supports fast, frequent access | Lower — limited by refresh & bus contention |
| Density | Low (larger cell size) | High (smaller cell, more bits/area) |
| Power | Higher static (leakage), no refresh needed | Needs periodic refresh (extra power/time) |
| Cost per bit | Expensive | Cheap |
| Volatility | Volatile (loses data without power) | Volatile (also needs refresh to retain data) |

Key Differences Explained

Latency: SRAM is faster because data is held in stable flip-flop circuits (immediate read/write, no capacitor charge/discharge). DRAM is slower since each access involves charging/discharging a capacitor and periodic refresh cycles to prevent data loss.

Bandwidth: SRAM, being on-chip and directly coupled to the core, uses wide, dedicated buses for high bandwidth. DRAM sits off-chip, connected via a narrower memory bus (e.g., DDR channels), limiting bandwidth and adding signal transmission delay.

Throughput: SRAM sustains more operations per cycle due to low latency and multiple simultaneous read/write ports in cache design. DRAM throughput is constrained by row/column access delays (RAS/CAS latency), refresh overhead, and shared bus contention across memory channels.

SRAM - Static RAM vs DRAM - Dynamic RAM:

Parameter ¦ SRAM ¦ DRAM

Cell Structure ¦ 6 transistors per bit, latch based, no refresh needed ¦ 1 transistor + 1 capacitor per bit, needs periodic refresh

Latency ¦ Very low: 0.5 - 5ns, 1-4 cycles for L1 ¦ High: 50 - 100ns, 200-300 CPU cycles

Bandwidth / Throughput ¦ Very high: can match CPU speed, multiple ports possible ¦ Lower: limited by refresh, row buffer, and shared bus

Density ¦ Low density, expensive ¦ High density, 6x to 8x denser than SRAM

Cost & Power ¦ High cost, low leakage but high static power ¦ Low cost, high dynamic power due to refresh

Usage ¦ L1, L2, L3 Cache ¦ Main Memory - RAM

Evaluation in System Design:

*1. For Cache Design - Why SRAM:*

Cache needs lowest latency and highest throughput to feed CPU without stalling. SRAM does not need refresh, so bandwidth is always available. It can provide data every cycle, achieving 2-3 TeraBytes/s in L1. Tradeoff is area, that is why L1 is only 32KB.

*2. For Main Memory Design - Why DRAM:*

Main memory needs large capacity at low cost. DRAM provides GBs of capacity cheaply. But its capacitor leaks charge, so controller must refresh every 64ms, which reduces effective throughput by 2-5%. Also DRAM access is two step - RAS and CAS, which increases latency. To hide this, modern DDR uses burst transfer, bank interleaving and prefetching to increase effective bandwidth.

3. Virtual Memory vs Paging vs Cache Memory

| Feature | Virtual Memory | Paging | Cache Memory |
| --- | --- | --- | --- |
| Purpose | Extends memory using secondary storage | Manages virtual-to-physical memory mapping | Speeds up CPU data/instruction access |
| Storage | RAM + Disk/SSD | RAM + Disk/SSD | SRAM |
| Access Latency | High when page fault occurs | High during page transfer | Very low |
| Bandwidth | Low compared with cache/RAM | Depends on storage and memory | Very high |
| Throughput | Can decrease due to page faults | Decreases when frequent swapping occurs | Improves CPU throughput |
| Main Benefit | Allows programs larger than physical RAM | Efficient memory allocation | Reduces CPU memory-access time |

Analysis

- Cache memory provides the lowest latency and is accessed frequently by the CPU.

- Paging divides virtual memory into fixed-size pages and maps them to physical frames.

- Virtual memory allows the system to use disk/SSD when RAM is insufficient, but this introduces very high latency during page faults.

- Therefore, cache memory has the greatest positive impact on CPU throughput, while excessive paging can significantly reduce system throughput.

### Virtual Memory (Paging) vs Cache Memory

| Feature | Cache Memory | Virtual Memory (Paging) |
| --- | --- | --- |
| Purpose | Speed up access to frequently used data | Extend usable address space beyond physical RAM |
| Managed by | Hardware (cache controller) | OS + MMU (hardware-assisted) |
| Access latency (hit) | Very low (~1–70 cycles) | Low (~100 ns, same as RAM) via TLB hit |
| Access latency (miss) | Goes to next cache level/RAM (~100+ cycles) | Page fault → disk/SSD access (~ms — orders of magnitude slower) |
| Unit of transfer | Cache line (32–128 bytes) | Page (typically 4 KB) |
| Throughput impact | Minor stalls on miss | Major stalls on page fault; can drastically drop throughput ("thrashing") |
| Backing store | Next cache level / RAM | Disk/SSD (swap space) |

Key point: Cache misses cost cycles; page faults cost milliseconds (disk I/O). Both use locality of reference, but virtual memory's penalty is far more severe — so OS-level page replacement algorithms (LRU, etc.) matter a lot for throughput, while cache misses are handled almost transparently by hardware.

Virtual Memory with Paging vs Cache Memory

Both use locality, but at different levels.

| Parameter Purpose Managed By Block Size Latency Throughput Impact | Cache Memory Hide gap between CPU and RAM Hardware Cache Controller Hardware + OS Cache line: 64 bytes Hit: 1-20 ns, Miss: 10-100 ns penalty Very high, GB/s to TB/s Improves IPC directly | Virtual Memory with Paging Hide gap between RAM and Disk, provide isolation. Hardware + OS MMU + Page Table Page: 4KB Page: 4KB - 4MB TLB Hit: 1 cycle, TLB Miss/Page Walk: 100-200 cycles, Page Fault: 10ms - Disk access. Low if page fault occurs, system throughput collapses due to disk I/O. If TLB hit rate is high, no impact. If page faults high, thrashing happens and throughput goes to near zero. |
| --- | --- | --- |

Key point: Cache miss costs 10x latency, Page fault costs 100,000x latency. So TLB is critical to make virtual memory viable.

# 4. NAND Flash vs DRAM

| # Feature | # NAND Flash | # DRAM |
| --- | --- | --- |
| # Latency | # Higher | # Much lower |
| # Bandwidth | # High for sequential operations | # Very high |
| # Throughput | # Good for bulk storage | # Excellent for frequent CPU access |
| # Volatility | # Non-volatile | # Volatile |
| # Refresh Required | # No | # Yes |
| # Write Performance | # Slower | # Faster |
| # Capacity | # Very high | # Moderate |
| # Cost per bit | # Low | # Higher |
| # Typical Use | # SSDs, storage | # Main memory |

# Analysis

# DRAM is suitable for the main-memory level because of its low latency and high bandwidth.

# NAND Flash has much higher access latency but provides high capacity and retains data without power.

# NAND Flash is therefore mainly used for secondary storage, while DRAM sits closer to the CPU.

### 4. NAND Flash vs DRAM

| Feature | NAND Flash | DRAM |
| --- | --- | --- |
| Latency | High (~10–100 µs) | Low (~50–100 ns) |
| Bandwidth | Lower (~500 MB/s–a few GB/s) | Higher (tens of GB/s) |
| Volatility | Non-volatile (retains data without power) | Volatile |
| Density/Cost | High density, cheap per GB | Lower density, expensive per GB |
| Endurance | Limited write/erase cycles (wear-out) | Effectively unlimited writes |
| Access granularity | Block/page-based (erase-before-write) | Byte-addressable |

Suitability: NAND Flash sits at the bottom of the hierarchy — used for persistent storage (SSDs), too slow for main memory duty. DRAM sits above it as main memory — fast enough to feed the CPU cache hierarchy but volatile, so it needs Flash/disk for persistence. Together they form the classic cache → DRAM → Flash/SSD hierarchy, balancing speed, cost, and persistence.

NAND Flash vs DRAM

| Parameter | DRAM | NAND Flash |
| --- | --- | --- |
| Latency | 50-100 ns, byte addressable | Read: 50-100 us, Write: 1-5 ms, block addressable |
| Bandwidth | High: 20-100 GB/s per channel - DDR5 | Low: 0.5-7 GB/s - limited by interface - NVMe |
| Endurance | Unlimited writes ∼10^15 | Limited: 1k-100k P/E cycles |
| Suitability | Main Memory - needs fast random access | Secondary Storage - SSD, for hierarchical level between DRAM and HDD |

## 5. MRAM vs RRAM

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Technology | Magnetic memory | Resistive memory |
| Access Time | Very fast | Fast |
| Read Latency | Low | Low |
| Write Latency | Low | Moderate, depending on design |
| Throughput | High | High |
| Energy Efficiency | High | Very high potential |
| Endurance | Very high | Generally lower than MRAM |
| Non-volatile | Yes | Yes |
| Scalability | Good | Excellent potential |
| Applications | Embedded memory, caches, IoT | Emerging storage and embedded memory |

# Analysis

# MRAM provides fast access, high endurance, and good throughput, making it attractive for replacing some SRAM/DRAM applications.

# RRAM uses changes in electrical resistance to store data and can offer low power consumption and high density.

# RRAM has strong potential for future high-density, energy-efficient memory systems.

# MRAM is generally more mature for applications requiring fast access and high endurance.

### MRAM vs RRAM

| Feature | MRAM | RRAM (ReRAM) |
| --- | --- | --- |
| Mechanism | Stores data via magnetic tunnel junction (spin polarity) | Stores data via resistance change in a dielectric |
| Access time | Fast (~ns, close to SRAM/DRAM) | Fast (~ns–tens of ns) |
| Throughput | High, good read/write symmetry | High, but write throughput can lag read |
| Energy efficiency | Low write energy, no refresh needed | Very low power, but write energy varies with resistance state |
| Endurance | Very high (10^12+ cycles) | Moderate-high (10^6–10^12, tech-dependent) |
| Non-volatility | Yes | Yes |
| Maturity/Density | More mature, moderate density | Still emerging, potentially higher density (crossbar-friendly) |

# MRAM vs RRAM:

Both are candidates for future unified memory.

| Parameter | MRAM (Magneto resistive RAM) | RRAM (Resistive RAM) |
| --- | --- | --- |
| Storage mechanism | Magnetic Tunnel Junction - spin | Filament formation by oxygen vacancies |
| Access time | Fastest among NVM: 10-30 ns read, 30-50 ns write - near DRAM | Moderate: 10-100 ns read, 100 ns - 1 us write |
| Throughput | High, can be byte addressable, good for cache replacement | Medium, crossbar array gives high density but sneak path limits throughput |
| Energy efficiency | High write energy, but zero standby power. Endurance >10^12 cycles | Very low write energy - pJ per bit, but endurance lower ∼10^6-10^9 cycles |
| Non-volatility | Yes | Yes |
| Suitability | Best for L3 cache or persistent main memory - SRAM replacement | Best for high density storage - DRAM and Flash replacement for IoT/AI edge |