SIMATS ENGINEERING

Department of Computer Science & Engineering ASSESSMENT TOOL 2-COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

COMPARATIVE ANALYSIS

1.Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

2.Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

3.Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

4.Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

5.Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

Code of Conduct:

I V.Rajitha(192311036) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: V.Rajitha

MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

# 1.L1, L2, and L3 Cache Comparison

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Inside CPU core | Near/inside CPU core | Shared among cores |
| Size | Smallest | Medium | Largest |
| Latency | Very low | Low | Higher than L1/L2 |
| Bandwidth | Highest | High | Lower than L1/L2 |
| Throughput | Highest | High | Moderate |
| Access Speed | Fastest | Fast | Slower |
| Main Purpose | Frequently used data/instructions | Backup for L1 | Shared backup for L1/L2 |

# Comparison of L1, L2, and L3 Cache Levels

| Parameter. Latency. Bandwidth. Throughput. Structure. | L1 Cache. 4–5 clock cycles (~1 ns). Highest (Terabytes/s per core). Maximum (serves core instructions per cycle). Core-private, split (I-Cache / D-Cache). | L2 Cache. 10–14 clock cycles (~3–4 ns). High (~1–2 TB/s per core). High (serves L1 misses). Core-private, unified. | L3 Cache. 30–50 clock cycles (~10– 15 ns). Moderate (~500–800 GB/s shared). Moderate (serves crosscore L2 misses). Shared across cores, unified. |
| --- | --- | --- | --- |

| Parameter | L1 Cache | L2 Cache | L3 Cache |  |
| --- | --- | --- | --- | --- |
| Latency | ~1–4 cycles (fastest) | ~10–20 cycles | ~30–70 cycles |  |
| Size | 16–64 KB (per core) | KB–1 MB (per core) | 4–64 MB (shared) |  |
| Bandwidth | Highest (closest to core) | Moderate-high | Lower than L1/L2 |  |
| Location | On-core, split I/D cache | On-core or percore | Shared across cores |  |
| Throughput | Highest ops/cycle | Moderate | Lowest of the three |  |

# L1, L2, and L3 Cache Levels Comparison

# 2. SRAM vs DRAM

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Full Form | Static RAM | Dynamic RAM |
| Latency | Very low | Higher |
| Bandwidth | High | High, but generally lower effective latency |
| Throughput | High | High |
| Refresh | Not required | Required |
| Cost | Expensive | Less expensive |
| Density | Lower | Higher |
| Typical Use | Cache memory | Main memory |

# SRAM vs. DRAM in Cache and Main Memory Design

| Characteristic. Latency. Bandwidth. Throughput. Cell Structure. Role in Hierarchy. | SRAM (Static RAM). Very Low (~1–10 ns). Extremely High (bus directly integrated with CPU logic). High per bit. 6 transistors (6T) per bit; static bistable latch. Primary choice for On-Die Caches (L1, L2, L3). | DRAM (Dynamic RAM). High (~50–100 ns). Moderate to High (limited by bus width and interface constraints). High aggregate (burst modes like DDR5). 1 transistor + 1 capacitor (1T1C); requires refresh cycles. Primary choice for Main System Memory. |
| --- | --- | --- |

SRAM vs DRAM

| Parameter | SRAM | DRAM |
| --- | --- | --- |
| Cell structure | 6-transistor flip-flop | 1 transistor + 1 capacitor |
| Latency | Very low (ns range, no refresh needed) | Higher (needs row activation, refresh cycles) |
| Bandwidth | High, but limited by area/cost at scale | High aggregate bandwidth (wide buses, burst modes) but higher per-access latency |
| Throughput | High for random access patterns | High for sequential/burst access; lower for random access due to row-buffer misses |
| Density/Cost | Low density, expensive per bit | High density, cheap per bit |
| Power | Static power (leakage) dominant | Needs periodic refresh, adding dynamic power overhead |

# 3. Virtual Memory with Paging vs Cache Memory

| Feature | Virtual Memory/Paging | Cache Memory |
| --- | --- | --- |
| Purpose | Extends available memory using storage | Speeds up CPU data access |
| Access Latency | Very high when page fault occurs | Very low |
| Storage | RAM + secondary storage | SRAM |
| Throughput | Lower during page faults | High |
| Speed | Slower | Much faster |
| Management | OS and memory management unit | Hardware/cache controller |
| Main Benefit | Allows large programs to run | Reduces CPU memory access time |

# Virtual Memory & Paging vs. Cache Memory

| Feature | Cache Memory System | Virtual Memory & Paging |
| --- | --- | --- |
| Access Latency | Nanoseconds (1 ns to 15 ns) | Microseconds to Milliseconds (Page Fault to SSD/HDD) |
| Throughput Impact | Prevents core starvation by keeping pipeline fed with fast SRAM hits. | Maximizes multiprogramming throughput by allowing virtual address mapping beyond physical RAM bounds. |
| Mechanism | Hardware-managed (SRAM tagging and hit/miss logic). | Hardware + OS-managed (TLB, Page Tables, Page Fault Handling). |
| Unit of Transfer | Cache Line (typically 64 Bytes). | Page Size (typically 4 KB to 2 MB/1 GB huge pages). |

# Virtual Memory (Paging) vs Cache Memory

| Aspect | Virtual Memory / Paging | Cache Memory |
| --- | --- | --- |
| Purpose | Extends addressable memory using disk/SSD as backing store; provides isolation | Reduces average latency to frequently used data from a slower main memory |
| Access latency on hit | TLB hit: near-zero overhead; page table walk on TLB miss: tens of cycles | Cache hit: single-digit to tens of cycles |
| Access latency on miss | Page fault: milliseconds (disk/SSD I/O) | Cache miss: goes to next level, tens to hundreds of cycles |
| Management | OS-managed (page tables, replacement policies like LRU/Clock) | Hardware-managed (replacement policies in hardware, transparent to software) |
| Granularity | Pages (typically 4 KB, or larger with huge pages) | Cache lines (typically 64 bytes) |
| Throughput impact | A page fault can stall a process for millions of cycles; amortized over long-running programs it's usually rare | Cache misses are frequent but individually cheap; aggregate effect dominates average CPI |

4. NAND Flash vs DRAM

| Feature | NAND Flash | DRAM |
| --- | --- | --- |
| Type | Non-volatile | Volatile |
| Latency | Higher | Lower |
| Bandwidth | High for sequential operations | Very high |
| Throughput | High, especially for storage | Very high for memory operations |
| Data Retention | Retains data without power | Loses data when power is removed |
| Refresh | Not required | Required |
| Main Use | SSDs, USB drives, storage | Main memory |
| Hierarchy Level | Secondary storage | Main memory |

# NAND Flash vs. DRAM

| Attribute. Latency. Bandwidth. Volatility. Access Granularity. | DRAM (Main Memory). ~50–100 ns. Very High (e.g., DDR5 up to 64 GB/s per channel). Volatile (loses data on power down). Byte/Word addressable. | NAND Flash (Secondary Storage). ~10–100 $\mu$s (Reads), ~1–10 ms (Writes). Moderate (PCIe Gen 5 NVMe up to 14 GB/s). Non-Volatile (retains data). Block/Page addressable (Requires erase-before-write). |
| --- | --- | --- |

NAND Flash vs DRAM

| Parameter | NAND Flash | DRAM |
| --- | --- | --- |
| Latency | Microseconds (read ~25– 100 μs including controller overhead; write/erase far slower) | Nanoseconds (tens of ns) |
| Bandwidth | Lower per-die, but aggregated across channels/dies in SSDs can be high for sequential I/O | Very high (GB/s per DIMM, tens of GB/s per channel) |
| Endurance | Limited program/erase cycles (wear-out); requires wear leveling | Effectively unlimited write endurance |
| Volatility | Non-volatile (retains data without power) | Volatile (requires refresh, loses data on power-off) |
| Density/Cost | Very high density, low cost per bit | Lower density than NAND, higher cost per bit |
| Access granularity | Page-based reads, blockbased erases (asymmetric read/write) | Byte/word addressable |

# 5. MRAM vs RRAM

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Full Form | Magnetoresistive RAM | Resistive RAM |
| Access Time | Very low | Very low |
| Latency | Low | Low |
| Throughput | High | High |
| Volatility | Non-volatile | Non-volatile |
| Energy Efficiency | High | Very high potential |
| Endurance | Very high | High |
| Main Principle | Magnetic states | Resistance states |
| Applications | Embedded memory, cache | Embedded memory, storage, AI hardware |

# MRAM vs. RRAM (Emerging NVM Technologies)

| Performance Parameter. Access Time / Latency. Throughput. Energy Efficiency. Working Principle. Primary Use Cases. | MRAM (Magnetoresistive RAM - STT/SOT). Fastest (~1–10 ns for reads/writes). High (comparable to SRAM speeds). High (Low dynamic write energy, zero static leakage). Magnetic Tunnel Junction (MTJ) spin-state resistance. Embedded SRAM/L3 cache replacement, persistent RAM. | RRAM (Resistive RAM / ReRAM). Moderate (~10–50 ns reads, slower writes). High for read-heavy vector-matrix operations. High (Ideal for low-power nonvolatile computing). Conductive filament formation in dielectric material. In-memory computing, Neuromorphic/AI accelerators. |
| --- | --- | --- |

# MRAM vs RRAM

| Parameter | MRAM (Magnetoresistive RAM) | RRAM (Resistive RAM) |
| --- | --- | --- |
| Storage mechanism | Magnetic tunnel junction (MTJ), stores state via electron spin orientation | Resistance change in a dielectric material via formation/rupture of conductive filaments |
| Access time | Fast (comparable to SRAM in STT-MRAM variants, ~ns range) | Fast, but generally slightly slower/more variable than MRAM; read/write asymmetry |
| Throughput | High, supports high endurance cycling well suited to frequent read/write | Good, though switching variability can limit sustained throughput |
| Endurance | Very high (10^12+ cycles for STT-MRAM), approaching DRAM/SRAM levels | Moderate-high, generally lower endurance than MRAM (10^6–10^10 cycles depending on material) |
| Energy efficiency | Low read energy; write energy moderate (currentdriven switching) | Very low read/write energy in ideal cases, but filament formation can require higher initial "forming" voltage |
| Non-volatility | Yes | Yes |
| Scalability/Density | Historically lower density than RRAM due to MTJ cell size | Simpler cell structure (can be denser, crossbarcompatible), better scaling potential |